# Temperature Sensor

## Features

### Number of Channels

Table 12.1	Number of Channels

| Temperature Sensor |  |
| --- | --- |
| Number of Channels | 1 |
| Name | OTS0 |

### Register Base Address

Table 12.2	Register Base Address

| Base Address Name | Base Address | Bus Group |
| --- | --- | --- |
| <OTSn_base> | FFC0 0200H | Peripheral Group 6L |

### Clock Supply

The clocks supplied to the temperature sensor are listed in the following table.

Table 12.3	Clock Supply

| Unit Name | Unit Clock Name | Clock Name |
| --- | --- | --- |
| OTS0 | Operation clock | CLK_LSB |
|  | Register access clock | CLK_LSB |

### Interrupt Requests

The temperature sensor interrupt requests are listed in the following table.

Table 12.4	Interrupt Requests

| Unit Interrupt Name | Unit Interrupt Signal Outline | Interrupt Number |
| --- | --- | --- |
| INTOTSOTE | Temperature sensor error interrupt | 660 |
| INTOTSOTI | Temperature measurement end interrupt | 661 |
| INTOTSOTULI | Triggered if the state machine changes the state due to a temperature rise or drop in the guaranteed temperature range | 662 |

Table 12.5	Alarm Notification

| Unit Interrupt Name | Outline | Connected to |
| --- | --- | --- |
| OTABE | Abnormal temperature error signal | ECM |

### External Input and Output Pins

This module has no external input/output pin.

## Overview

### Functional Overview

The following describes features of the temperature sensor.

Temperature data register

A temperature data register stores a temperature measurement value.

Temperature measurement mode

Single measurement mode: Used to measure temperature only once.

Continuous measurement mode: Used to measure temperature continuously.

Supporting temperature measurement end interrupt

Each time temperature measurement ends, the temperature sensor can generate an interrupt request (INTOTSOTI) to be sent to the INTC.

Supporting abnormal temperature error signal and temperature rise / drop interrupt

Six temperature threshold values (high-temperature border AU > high-temperature border AL > high-temperature border BU > high-temperature border BL > low-temperature border AU > low-temperature border AL) should be set in advance. Furthermore, the temperature sensor has four temperature states (high temperature A, high temperature B, normal temperature, and low temperature A). Temperature state is updated at each temperature measurement according to six temperature threshold values. The following describes conditions for generating a abnormal temperature error  signal and a temperature rise/drop interrupt.

An abnormal temperature error  signal (OTABE) is output at a transition from a state other than high temperature A to high temperature A or at a transition from a state other than low temperature A to low temperature A.

- A temperature rise/drop interrupt (INTOTSOTULI) is output at a transition from high temperature A to high temperature B or normal temperature or at a transition from high temperature B to normal temperature, that is when temperature drops.

- A temperature rise/drop interrupt (INTOTSOTULI) is output at a transition from low temperature A to high temperature B or normal temperature or at a transition from normal temperature to high temperature B, that is, when temperature rises.

Temperature status can be monitored by reading the temperature status register.

- Diagnosis function

When temperature is measured in continuous measurement mode, if the difference between the measured value and the previously measured value is larger than the specified temperature difference limit value, a temperature sensor error interrupt (INTOTSOTE) is output.

- Reducing errors by temperature correction

Three temperature correction coefficients are stored during device manufacturing in coefficient registers A to C.

### Block Diagram

Figure12.1 shows a block diagram of the temperature sensor.

Figure 12.1	Temperature Sensor Block Diagram

Figure 12.2	Temperature Sensor Block Diagram [For internal use only]

## Register

### List of registers

The following table lists the registers of the temperature sensor.

<OTSn_base> is defined in Section 13.1.2 Register Base Address.

Table 12.6	List of Registers

| Unit Name | Register Name | Symbol | Address | Access Size |
| --- | --- | --- | --- | --- |
| OTSn | Temperature measurement start control register | OTSnOTSTCR | <OTSn_base> + 00H | 8 |
| OTSn | Temperature measurement end control register | OTSnOTENDCR | <OTSn_base> + 04H | 8 |
| OTSn | Temperature sensor control register | OTSnOTCR | <OTSn_base> + 08H | 8 |
| OTSn | Temperature sensor flag clear register | OTSnOTFCR | <OTSn_base> + 0CH | 8 |
| OTSn | Temperature sensor flag register | OTSnOTFR | <OTSn_base> + 10H | 8 |
| OTSn | Temperature status register | OTSnOTSTR | <OTSn_base> + 14H | 8 |
| OTSn | Temperature data register | OTSnOTDR | <OTSn_base> + 18H | 16 |
| OTSn | High-temperature border AU register | OTSnHTBRAU | <OTSn_base> + 1CH | 16 |
| OTSn | High-temperature border AL register | OTSnHTBRAL | <OTSn_base> + 20H | 16 |
| OTSn | High-temperature border BU register | OTSnHTBRBU | <OTSn_base> + 24H | 16 |
| OTSn | High-temperature border BL register | OTSnHTBRBL | <OTSn_base> + 28H | 16 |
| OTSn | Low-temperature border AU register | OTSnLTBRAU | <OTSn_base> + 2CH | 16 |
| OTSn | Low-temperature border AL register | OTSnLTBRAL | <OTSn_base> + 30H | 16 |
| OTSn | Temperature difference limiting register | OTSnTDLR | <OTSn_base> + 34H | 16 |
| OTSn | Coefficient A register | OTSnCOEFFRA | <OTSn_base> + 38H | 16 |
| OTSn | Coefficient B register | OTSnCOEFFRB | <OTSn_base> + 3CH | 16 |
| OTSn | Coefficient C register | OTSnCOEFFRC | <OTSn_base> + 40H | 16 |
| OTSn | Self-diagnosis control register | OTSnSDIAGCTL | <OTSn_base> + A0H | 8 |
| OTSn | Coefficient select register [For internal use only] | OTSnCOEFFSR | <OTSn_base> + 80H | 8 |
| OTSn | Coefficient A tuning register [For internal use only] | OTSnCOEFFTUNERA | <OTSn_base> + 84H | 16 |
| OTSn | Coefficient B tuning register [For internal use only] | OTSnCOEFFTUNERB | <OTSn_base> + 88H | 16 |
| OTSn | Coefficient C tuning register [For internal use only] | OTSnCOEFFTUNERC | <OTSn_base> + 8CH | 16 |
| OTSn | Test register A [For internal use only] | OTSnTSTRA | <OTSn_base> + 90H | 8 |
| OTSn | Test register B [For internal use only] | OTSnTSTRB | <OTSn_base> + 94H | 8 |
| OTSn | Test register C [For internal use only] | OTSnTSTRC | <OTSn_base> + 98H | 8 |
| OTSn | Test register D [For internal use only] | OTSnTSTRD | <OTSn_base> + 9CH | 8 |

Note: (n = 0)

### Reset of Registers

Table 12.7	Register Reset Conditions

| Register Name |  | Reset Source |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Power On Reset | System Reset 1 | System Reset 2 | Application Reset | Deep STOP reset | Module Reset | JTAG Reset |
| All registers | √ | √ | √ | √ | √ | — | — |

### OTS0OTSTCR — Temperature Measurement Start Control Register

OTS0OTSTCR is an 8-bit write-only control register to start the temperature sensor.

| Access: |  | This register can be written in 8-bit units. |
| --- | --- | --- |
| Address: |  | <OTSn_base> + 00H |
| Value after reset: |  | 00H |

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | — | — | — | — | — | — | — | OTST |
| Value after reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| R/W | R | R | R | R | R | R | R | W |

Table 12.8	OTS0OTSTCR Register Contents

| Bit Position | Bit Name | Function |
| --- | --- | --- |
| 7 to 1 | — | Reserved When read, the value after reset is returned. When writing, write the value after reset. |
| 0 | OTST | Temperature Measurement Start Condition for starting temperature measurement by OTST. Write 1 to OTST when OTACT = 0 |

### OTS0OTENDCR — Temperature Measurement End Control Register

OTS0OTENDCR is an 8-bit write-only control register to terminate the temperature sensor.

| Access: |  | This register can be written in 8-bit units. |
| --- | --- | --- |
| Address: |  | <OTSn_base> + 04H |
| Value after reset: |  | 00H |

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | — | — | — | — | — | — | — | OTEND |
| Value after reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| R/W | R | R | R | R | R | R | R | W |

Table 12.9	OTS0OTENDCR Register Contents

| Bit Position | Bit Name | Function |
| --- | --- | --- |
| 7 to 1 | — | Reserved When read, the value after reset is returned. When writing, write the value after reset. |
| 0 | OTEND | Temperature Measurement End Condition for terminating temperature measurement by OTEND. Write 1 to OTEND and then wait for 2 cycles of the OTS clock. |

### OTS0OTCR — Temperature Sensor Control Register

OTS0OTCR is an 8-bit readable/writable register to control the temperature sensor.

| Access: |  | This register can be read or written in 8-bit units. |
| --- | --- | --- |
| Address: |  | <OTSn_base> + 08H |
| Value after reset: |  | 00H |

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | — | — | — | OTEE | OTULIE | OTABEE | SDE | OTMD |
| Value after reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| R/W | R | R | R | R/W | R/W | R/W | R/W | R/W |

Table 12.10	OTS0OTCR Register Contents

| Bit Position | Bit Name | Function |
| --- | --- | --- |
| 7 to 5 | — | Reserved When read, the value after reset is returned. When writing, write the value after reset. |
| 4 | OTEE | Temperature Sensor Error Enable 0: Disabled 1: Enabled When OTMD = 0 (single measurement mode) or SDE = 0 (diagnosis disabled), no temperature sensor error is generated regardless of the OTEE setting. |
| 3 | OTULIE | Temperature Rise / Drop Interrupt Enable 0: Disabled 1: Enabled |
| 2 | OTABEE | Temperature Alarm Error Enable 0: Disabled 1: Enabled |
| 1 | SDE | Diagnosis Enable 0: Disabled 1: Enabled When OTMD = 0 (single measurement mode), diagnosis is not performed regardless of the SDE setting. |
| 0 | OTMD | Temperature Measurement Mode 0: Single temperature measurement mode 1: Continuous temperature measurement mode |

CAUTION

To prevent malfunction, set OTS0OTCR while the OTACT bit in OTS0OTFR is 0.

### OTS0OTFCR — Temperature Sensor Flag Clear Register

OTS0OTFCR is an 8-bit write-only register to clear the OTS0OTFR register.

| Access: |  | This register can be written in 8-bit units. |
| --- | --- | --- |
| Address: |  | <OTSn_base> + 0CH |
| Value after reset: |  | 00H |

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | — | — | — | — | — | SDERC | — | OTFC |
| Value after reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| R/W | R | R | R | R | R | W | R | W |

Table 12.11	OTS0OTFCR Register Contents

| Bit Position | Bit Name | Function |
| --- | --- | --- |
| 7 to 3 | — | Reserved When read, the value after reset is returned. When writing, write the value after reset. |
| 2 | SDERC | Diagnosis Error Clear Writing 0: Does not clear the diagnosis error. Writing 1: Clears the diagnosis error. |
| 1 | — | Reserved When read, the value after reset is returned. When writing, write the value after reset. |
| 0 | OTFC | Temperature Measurement End Flag Clear Writing 0: Does not clear the temperature measurement end flag. Writing 1: Clears the temperature measurement end flag. |

### OTS0OTFR — Temperature Sensor Flag Register

OTS0OTFR is an 8-bit read-only register to indicate temperature sensor flags.

| Access: |  | This register is a read-only register that can be read in 8-bit units. |
| --- | --- | --- |
| Address: |  | <OTSn_base> + 10H |
| Value after reset: |  | 00H |

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | — | — | — | — | — | SDER | OTACT | OTF |
| Value after reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| R/W | R | R | R | R | R | R | R | R |

Table 12.12	OTS0OTFR Register Contents

| Bit Position | Bit Name | Function |
| --- | --- | --- |
| 7 to 3 | — | Reserved When read, the value after reset is returned. |
| 2 | SDER | Diagnosis Error Setting condition A diagnosis error occurred. Clearing condition Write SDERC to 1 in OTS0OTFCR |
| 1 | OTACT | Temperature Sensor Status 0: The temperature sensor is in the idle state. 1: The temperature sensor is operating. |
| 0 | OTF | Temperature Measurement End Flag Setting condition A temperature measurement value is written to OTS0OTDR Clearing condition Write OTFC to 1 in OTS0OTFCR. Reading of the OTS0OTDR register. |

CAUTION

SDER is updated when a temperature measurement value is written to OTS0OTDR.

### OTS0OTSTR — Temperature Status Register

OTS0OTSTR is an 8-bit read-only register to indicate temperature sensor status.

| Access: |  | This register is a read-only register that can be read in 8-bit units. |
| --- | --- | --- |
| Address: |  | <OTSn_base> + 14H |
| Value after reset: |  | 00H |

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | — | — | — | — | — | — | TSTAT[1:0] |  |
| Value after reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| R/W | R | R | R | R | R | R | R | R |

Table 12.13	OTS0OTSTR Register Contents

| Bit Position | Bit Name | Function |
| --- | --- | --- |
| 7 to 2 | — | Reserved When read, the value after reset is returned. |
| 1 to 0 | TSTAT[1:0] | Temperature Status 00: Normal temperature (value after reset ) 01: High temperature B 10: High temperature A 11: Low temperature A When temperature is measured, temperature status is determined by the temperature measurement value and the status is reflected in TSTAT[1:0]. Figure 13.3 shows status transitions of TSTAT. Table 13.12 shows temperature alarm error and temperature rise/drop interrupt generating conditions. |

CAUTION

The TSTAT[1:0] bits are updated when a temperature measurement value is written to OTS0OTDR.

Figure 12.3	Temperature Status and Transition States

Table 12.14 Conditions for Generating Abnormal Temperature Error and Temperature Rise/Drop Interrupts

|  |  | After Status Transition |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | High temp. A | High temp. B | Normal temp. | Low temp. A |
| Before status transition | High temp. A | N/A | INTOTSOTULI | INTOTSOTULI | OTABE |
|  | High temp. B | OTABE | N/A | INTOTSOTULI | OTABE |
|  | Normal temp. | OTABE | INTOTSOTULI | N/A | OTABE |
|  | Low temp. A | OTABE | INTOTSOTULI | INTOTSOTULI | N/A |

[Legend]

OTABE: Occurrence of temperature alarm error

INTOTSOTULI: Occurrence of temperature rise or drop interrupt

N/A: Neither OTABE nor INTOTSOTULI occurs.

### OTS0OTDR — Temperature Data Register

OTS0OTDR is a 16-bit read-only register that stores a temperature measurement value.

| Access: |  | This register is a read-only register that can be read in 16-bit units. |
| --- | --- | --- |
| Address: |  | <OTSn_base> + 18H |
| Value after reset: |  | 0000H |

|  | Signed fixed-point format |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
|  | OTD*1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Value after reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| R/W | R | R | R | R | R | R | R | R | R | R | R | R | R | R | R | R |
|  | ↑ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Decimal point position |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - OTD[15]: Sign bit (always 0) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 12.15	OTS0OTDR Register Contents

| Bit Position | Bit Name | Function |
| --- | --- | --- |
| 15 to 0 | OTD | Temperature Data Value These bits specify a value for temperature data. OTD[15] is always 0. |

Table 12.16 Temperature guideline

### OTS0HTBRmn — High-Temperature Border mn Register (m = A or B, n = U or L)

OTS0HTBRmn is a 16-bit readable/writable register to specify the temperature border of high temperature m.

| Access: |  | This register can be read or written in 16-bit units. |
| --- | --- | --- |
| Address: |  | OTS0HTBRAU: <OTSn_base> + 1CH , OTS0HTBRAL: <OTSn_base> + 20H |
|  |  | OTS0HTBRBU: <OTSn_base> + 24H , OTS0HTBRBL: <OTSn_base> + 28H |
| Value after reset: |  | 0000H |

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | HTBmn*1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Value after reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| R/W | R | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |
|  | - HTBmn[15]: Sign bit (always 0) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 12.17	OTS0HTBRmn Register Contents

| Bit Position | Bit Name | Function |
| --- | --- | --- |
| 15 to 0 | HTBmn | High-Temperature Border mn These bits specify the high-temperature border mn. Specify it so that the following condition is met. OTS0HTBRAU > OTS0HTBRAL > OTS0HTBRBU > OTS0HTBRBL > OTS0LTBRAU > OTS0LTBRAL The HTBmn format is the same as the OTS0OTDR format. HTBmn[15] is always 0. For details, see Section 13.3.8, OTS0OTSTR — Temperature Status Register. |

CAUTION

To prevent malfunction, set OTS0HTBRmn while the OTACT bit in OTS0OTFR is 0.

### OTS0LTBRAn — Low-Temperature Border An Register (n = U or L)

OTS0LTBRAn is a 16-bit readable/writable register to specify the temperature border of low temperature A.

| Access: |  | This register can be read or written in 16-bit units. |
| --- | --- | --- |
| Address: |  | OTS0LTBRAU: <OTSn_base> + 2CH , OTS0LTBRAL: <OTSn_base> + 30H |
| Value after reset: |  | 0000H |

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LTBAn*1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Value after reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| R/W | R | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |
|  | - LTBAn[15]: Sign bit (always 0) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 12.18	OTS0LTBRAn Register Contents

| Bit Position | Bit Name | Function |
| --- | --- | --- |
| 15 to 0 | LTBAn | Low-Temperature Border An These bits specify the low-temperature border An. Specify it so that the following condition is met. OTS0HTBRAU > OTS0HTBRAL > OTS0HTBRBU > OTS0HTBRBL > OTS0LTBRAU > OTS0LTBRAL The LTBAn format is the same as the OTS0OTDR format. LTBAn[15] is always 0. For details, see Section 13.3.8, OTS0OTSTR — Temperature Status Register. |

CAUTION

To prevent malfunction, set OTS0LTBRAn while the OTACT bit in OTS0OTFR is 0.

### OTS0TDLR — Temperature Difference Limiting Register

OTS0TDLR is a 16-bit readable/writable register to specify the temperature difference limit value for diagnosis.

| Access: |  | This register can be read or written in 16-bit units. |
| --- | --- | --- |
| Address: |  | <OTSn_base> + 34H |
| Value after reset: |  | 7FFFH |

|  | Signed fixed-point format |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
|  | TDL*1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Value after reset | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| R/W | R | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |
|  | ↑ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Decimal point position |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - TDL[15]: Sign bit (always 0) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 12.17	OTS0TDLR Register Contents

| Bit Position | Bit Name | Function |
| --- | --- | --- |
| 15 to 0 | TDL | Temperature Difference Limit Value These bits specify a value for limiting temperature difference between present temperature measurement value and previous temperature measurement value in continuous measurement mode. When the following condition is met, SDER (diagnosis error) is set to 1. OTS0TDLR < ¦ present temperature measurement value – previous temperature measurement value ¦ TDL[15] is always 0. |

CAUTION

To prevent malfunction, set OTS0TDLR while the OTACT bit in OTS0OTFR is 0.

### OTS0COEFFRn — Coefficient n Register (n = A, B, C)

OTS0COEFFRn is a 16-bit read-only register to store coefficients for correction calculation.

Coefficients are stored in OTS0COEFFRn during device manufacturing.

| Access: |  | This register is a read-only register that can be read in 16-bit units. |
| --- | --- | --- |
| Address: |  | OTS0COEFFRA: <OTSn_base> + 38H , OTS0COEFFRB: <OTSn_base> + 3CH |
|  |  | OTS0COEFFRC: <OTSn_base> + 40H |
| Value after reset: |  | The initial value will be stored at delivery inspection for each devices |

|  | Signed fixed-point format |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
|  | COEFFn*1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Value after reset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W | R | R | R | R | R | R | R | R | R | R | R | R | R | R | R | R |
|  |  |  |  |  |  |  |  |  |  |  |  | ↑ |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Decimal point position |  |  |  |  |
|  | - COEFFn[15]: Sign bit (always 0) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 12.18	OTS0COEFFRn Register Contents

| Bit Position | Bit Name | Function |
| --- | --- | --- |
| 15 to 0 | COEFFn | Coefficients Value for correction calculation These bits specify a value for coefficients Value. COEFFn[15] is always 0. |

- Reducing errors by temperature correction
Three temperature correction coefficients are stored during device manufacturing in coefficient registers A to C. 
Errors in temperature measurement values can be reduced by making a correction by using the temperature correction calculation formula that uses the temperature correction coefficients. However, temperature corrections must be made by the CPU. By using this process, the Temperature Border setting can be improved.

- Correction calculation formula: T = AX2 + BX + C

- T: Temperature value after correction

- X: Temperature value before correction

A to C: Coefficients A to C

### OTS0COEFFSR — Coefficient Select Register [For internal use only]

OTS0COEFFSR is register for selecting the coefficient for the correction operation.

It is possible to readable/writable only in the test mode.

| Access: |  | This register can be read or written in 8-bit units. |
| --- | --- | --- |
| Address: |  | <OTSn_base> + 80H |
| Value after reset: |  | 00H |

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | — | — | — | — | — | — | — | COEFFS |
| Value after reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| R/W | R | R | R | R | R | R | R | R/W |

Table 12.19	OTS0COEFFSR Register Contents

| Bit Position |  | Bit Name | Function |
| --- | --- | --- | --- |
| 7 to 1 |  | — | Reserved When read, the value after reset is returned. When writing, write the value after reset. |
| 0 |  | COEFFS | Coefficient selection 0: Select COEFFRn (Trimming value) 1: Select COEFFTUNERn (Tuning coefficients) |

### OTS0COEFFTUNERn — Coefficient n Register (n = A, B, C) [For internal use only]

OTS0COEFFTUNERn is register for tuning the coefficients for correction operation

It is possible to readable/writable only in the test mode.

| Access: |  | This register can be read or written in 16-bit units. |
| --- | --- | --- |
| Address: |  | OTS0COEFFTUNERA: <OTSn_base> + 84H , OTS0COEFFTUNERB: <OTSn_base> + 88H , OTS0COEFFTUNERC: <OTSn_base> + 8CH |
| Value after reset: |  | 0000H |

| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | COEFFTUNEn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Value after reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

Table 12.20	OTS0COEFFTUNERn Register Contents

| Bit Position | Bit Name | Function |
| --- | --- | --- |
| 15 to 0 | COEFFTUNEn | Tuning coefficient n Specify any factor in the test mode. OTS0COEFFTUNEn and OTS0COEFFRn is the same format. |

### OTS0TSTRA — Test Register A [For internal use only]

OTS0TSTRA is a register for setting the test function.

It is possible to readable/writable only in the test mode.

| Access: |  | This register can be read or written in 8-bit units. |
| --- | --- | --- |
| Address: |  | <OTSn_base> + 90H |
| Value after reset: |  | 00H |

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | OVSR | EXANMD | OCCHP | AMPCHP | TEST[3:0] |  |  |  |
| Value after reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

Table 12.23	OTS0TSTRA Register Contents

| Bit Position | Bit Name | Function |
| --- | --- | --- |
| 7 | OVSR | Over-sampling rate 0: About 51.28 kHz (1/19.5 µsec) (Normal) 1: 312.5 kHz (for time reduction of the A/D accuracy test of ΔΣADC) Change the frequency of the OTS clock (DSCLK terminal of HM) to be generated by the SM. |
| 6 | EXANMD | External analog input mode 0: BGR connects the output of the (sensor) to the input of the ΔΣADC of the internal temperature sensor (usually) 1: Connect the analog input of the SAR-ADC to the input of ΔΣADC of the internal temperature sensor (A / D accuracy test of ΔΣADC) It is used to implement the A / D accuracy test of ΔΣADC. |
| 5 | OCCHP | ΔΣADC chopper 0: ON (Normal) 1: OFFOCCHP_EN_N terminal of HM = OCCHP. |
| 4 | AMPCHP | ΔΣADC integrator AMP chopper 0: ON (Normal) 1: OFF AMPCHP_EN_N terminal of HM = AMPCHP. |
| 3 to 0 | TEST[3:0] | ΔΣADC failure detection mode 0H: Normal Other: Fault detection test mode TEST [3: 0] terminal of HM = TEST [3:0]. |

### OTS0TSTRB — Test Register B [For internal use only]

OTS0TSTRB is a register for the test missing in temperature.

It is possible to readable/writable only in the test mode.

| Access: |  | This register can be read or written in 8-bit units. |
| --- | --- | --- |
| Address: |  | <OTSn_base> + 94H |
| Value after reset: |  | 00H |

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | — | — | — | — | — | — | ONDO[1:0] |  |
| Value after reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| R/W | R | R | R | R | R | R | R/W | R/W |

Table 12.22	OTS0TSTRB Register Contents

| Bit Position | Bit Name | Function |
| --- | --- | --- |
| 7 to 2 | — | Reserved When read, the value after reset is returned. When writing, write the value after reset. |
| 1 to 0 | ONDO[1:0] | BGR part PTAT voltage adjustment 0H: Normal Other: BGR part current switching test mode TEST_ONDO [1: 0] terminal of HM = ONDO [1: 0]. |

### OTS0TSTRC — Test Register C [For internal use only]

OTS0TSTRC is a register for setting the BGR unit consumption current switching.

It is possible to readable/writable only in the test mode.

| Access: |  | This register can be read or written in 8-bit units. |
| --- | --- | --- |
| Address: |  | <OTSn_base> + 98H |
| Value after reset: |  | 00H |

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | — | — | — | — | IBIAS[3:0] |  |  |  |
| Value after reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| R/W | R | R | R | R | R/W | R/W | R/W | R/W |

Table 12.23	OTS0TSTRC Register Contents

| Bit Position | Bit Name | Function |
| --- | --- | --- |
| 7 to 4 | — | Reserved When read, the value after reset is returned. When writing, write the value after reset. |
| 3 to 0 | IBIAS[3:0] | BGR part consumption power switching 0H: Normal Other: Reference current switching test mode of the constant current source TEST_IBIAS [3: 0] terminal of HM = IBIAS [3: 0]. |

### OTS0TSTRD — Test Register D [For internal use only]

OTS0TSTRD is a register to monitor the internal timing signal.

It is possible to readable only in the test mode.

| Access: |  | This register is a read-only register that can be read in 8-bit units. |
| --- | --- | --- |
| Address: |  | <OTSn_base> + 9CH |
| Value after reset: |  | 00H |

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | — | — | — | — | — | — | — | FFCHK |
| Value after reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| R/W | R | R | R | R | R | R | R | R |

Table 12.24	OTS0TSTRD Register Contents

| Bit Position | Bit Name | Function |
| --- | --- | --- |
| 7 to 1 | — | Reserved When read, the value after reset is returned. |
| 0 | FFCHK | Monitoring of the internal timing signal. FFTESTCHECK terminal is monitored. |

### OTS0SDIAGCTL - Self-Diagnosis Control Register

OTS0SDIAGCTL is an 8-bit readable/writable register to control the input of AD converter for self-diagnosis.

| Access: |  | This register can be read or written in 8-bit units. |
| --- | --- | --- |
| Address: |  | <OTSn_base> + A0H |
| Value after reset: |  | 00H |

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | — | — | — | — | — | — | ADCIN[1:0] |  |
| Value after reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| R/W | R | R | R | R | R | R | R/W | R/W |

Table 12.25	OTS0SDIAGCTL Register Contents

| Bit Position | Bit Name | Function |
| --- | --- | --- |
| 7 to 2 | — | Reserved When read, the value after reset is returned. |
| 1 to 0 | ADCIN[1:0] | AD Converter Input Control for Self-Diagnosis. 00: No pull-up/pull-down (normal operation). 01: Pull-down. 10: Pull-up. 11: Setting prohibited. |

## Operation

### Temperature Measurement Sequence

The temperature sensor is equipped with a Single measurement mode and a Continuous measurement mode.

The measurement start trigger and Measurement end trigger of each mode is shown below.

Table 12.27	the Measurement Start/End Trigger

| Measurement Mode | Measurement Start Trigger | Measurement End Trigger |
| --- | --- | --- |
| Single measurement mode OTS0OTCR.OTMD = 0 | Write 1B in OTS0OTSTCR.OTST. | 1) Automatic stop after one temperature measurement ends 2) Write 1B in OTS0OTENDCR.OTEND.*1 *2 |
| Continuous measurement mode OTS0OTCR.OTMD = 1 | Write 1B in OTS0OTSTCR.OTST. | Write 1B in OTS0OTENDCR.OTEND.*1 |

- If the temperature measurement has been stopped by the OTS0OTENDCR.OTEND, data during the measurement will be discarded.

If the temperature measurement has been stopped by the OTS0OTENDCR.OTEND before the first measurement has been completed the measurement operation will not output the measurement result at all.

The time until the temperature data is output requires 512 states (9.984msec. OTS0TSTRA.OVSR = 1 set at the time of, 1.6384msec [For internal use only]) of the OTS clock to convert the measured temperature to digital data.

There is a measurement stabilization period of 3 OTS clock states before the temperature is sampled after the measurement process is started by writing to the OTS0OTSSTCR.OTST bit.

In Single measurement mode, to update the temperature measurement result requires 515 OTS clock states, including the measurement preparation period, until the temperature measurement operation ends.

In Continuous measurement mode, 515 of the OTS clock, including the 3 state measurement stabilization period, are required only for the first temperature measurement, the second and subsequent updates of the temperature measurement result requires 512 states, and the measurement operation is repeated until 1 is written to OTENDCR.OTEND.

### Examples of Temperature Measurement Operation

The following figure shows temperature measurement operation in single measurement mode.

Figure 12.4	Example of Temperature Measurement Operation (Single Measurement Mode)

NOTE

If “1” is written to OTENDCR.OTEND at the time of the internal cycles 511 or 512.
INTOTSOTI is issued and OTS0OTFR.OTF is set after updating the temperature measurement result as with the case of automatic stop after the temperature measurement.

DSOUT is the operation clock of the temperature sensor obtained by dividing CLK_LSB.

The period of DSOUT is 19.5 us ( When CLK_LSB 40MHz )

The following figure shows temperature measurement operation in continuous measurement mode. There is no internal stabilization time in the second and subsequent temperature measurements.

Figure 12.5	Example of Temperature Measurement Operation (Continuous Measurement Mode)

### Temperature Measurement End Interrupt Request

The temperature sensor can generate a temperature measurement end interrupt request (INTOTSOTI) to be sent to the INTC. At this time, the OTS0OTFR.OTF is set.

The table below shows the assertion condition and negated conditions of INTOTSOTI and OTS0OTFR.OTF.

Table 12.28	the Temperature Measurement End Conditions

| Measurement End Notification Signal | Assert Conditions | Negated Conditions | Enable Control |
| --- | --- | --- | --- |
| Temperature measurement end interrupt (INTOTSOTI) | Each time temperature measurement ends | After the assertion, one cycle after the register access clock | No enable control bit. |
| Temperature measurement end flag (OTS0OTFR.OTF) | Each time temperature measurement ends | 1) Reading of the OTS0OTDR register 2) Write 1B in OTS0OTFCR.OTFC | No enable control bit. |

For the OTS0OTFR.OTF bit if the assertion conditions and negated conditions conflict, negated condition takes precedence.

If the temperature measurement has been stopped by the OTS0OTENDCR.OTEND, neither the INTOTSOTI nor the OTS0OTFR.OTF will occur.

However, after writing "1" to the OTS0OTENDCR.OTEND, when the measurement is within 2 OTS clock cycles of being completed, the measurement result is updated and the INTOTSOTI and OTS0OTFR.OTF are asserted.

Figure 12.6	Example of Occurrence of a Temperature Measurement End Interrupt

### Temperature Alarm Error and Temperature Rise / Drop Interrupt and Temperature Sensor Error Interrupt Requests

The temperature sensor can generate a temperature rise or drop interrupt request (INTOTSOTULI) a temperature sensor error interrupt request (INTOTSOTE) and an abnormal temperature error (OTABE) to be sent to the ECM. Setting OTULIE in OTS0OTCR to 1 enables INTOTSOTULI. Setting OTULIE to 0 disables INTOTSOTULI. Setting OTEE in OTS0OTCR to 1 enables INTOTSOTE. Setting OTEE to 0 disables INTOTSOTE. Setting OTABEE in OTS0OTCR to 1 enables OTABE. Setting OTABEE to 0 disables OTABE.

The table below shows the assertion condition and negated conditions of INTOTSOTULI and OTABE.

Table 12.29	the Temperature Sensor Interrupts

| Temperature State Transition Notification Signal | Assert Conditions | Negated Conditions | Enable Control |
| --- | --- | --- | --- |
| Temperature rise or drop interrupt request (INTOTSOTULI) | Temperature state transition from a state other than the high-temperature B to high temperature B. Temperature state transition from a state other than the normal temperature to normal temperature. | After the assertion, one cycle after the register access clock | OTS0OTCR.OTULIE |
| Abnormal Temperature error (OTABE) | Temperature state transition from a state other than the high temperature A to high temperature A. Temperature state transition from a state other than the low temperature A to low temperature A. | After the assertion, one cycle after the register access clock | OTS0OTCR.OTABEE |
| Temperature sensor error interrupt (INTOTSOTE) | After the OTDR register is updated, when the difference between the temperature measurements exceeds a set value TDL. | After the assertion, one cycle after the register access clock | OTS0OTCR.OTEE and OTS0OTCR.SDE |
| Diagnostic error flag (OTS0OTFR.SDER) | After the OTDR register is updated, when the difference between the temperature measurements exceeds a set value TDL. | Write 1B in OTS0OTFCR.SDERC | OTS0OTCR.SDE |

Figure 12.7	Example of Occurrence of a Temperature Alarm Error and a Temperature Rise / Drop Interrupt and a Temperature Sensor Error Interrupt

### Self-diagnosis

Every time the temperature measurement is ended, the previous measurement result and the current measurement result are compared, and when the temperature change reaches the TDLR register TDL
Self-diagnostic error flag (SDER) is set when the threshold value set in At the same time, temperature sensor
If temperature sensor output is enabled, a temperature sensor error (OTE) will be generated.

SDER flag and OTE generation condition

1.5.6. クロック停止制御

The clock input in the soft macro can be controlled by the pin input.
The terminals that control clock supply are mstp_p and mstp_ot.

| Terminal state | Operation explanation |
| --- | --- |
| mstp_p＝mstp_ot＝low | Temperature measurement operation is possible. |
| mstp_p＝high | APB I/F The control circuit stops. (Ignore even APB access, temperature measurement operation is not possible) |
| mstp_ot =high | Timing control circuit and signal processing block for temperature measurement are stopped. (Temperature measurement operation is not possible) |

## DFT information [For internal use only]

There are no non-scan FF in SM.

HM(ARV28TPS0000AKGLTS2K30T55A0) is not target of scan.

It is controlled by ARV28TPS0000AKGLTS2K30T55A0_dft_wrapper module in DFT mode.

And this HM is checked by direct test for DFT mode.

## Test Function [For internal use only]

### Temperature Sensor the Direct Test Specification

#### Block Diagram

Figure 12.8	Block Diagram (Test Function)

#### Details of Registers

GPTR0 (the time of a TESTEMPS mode)

It's effective at SUBMD [20] (TESTEMPS) = 1.

GPTR1 (the time of a TESTEMPS mode)

It's effective at SUBMD [20] (TESTEMPS) = 1.

GPTR0 (the time of a TESTEMPS mode)

GPTR1 (the time of a TESTEMPS mode)

#### the Matrix of Direct Test Mode

Failure of a temperature sensor is detected or in order to check the simple substance accuracy of the Delta-Sigma ADC or a temperature sensor soft macro, it is necessary to measure by the item of the following table.

| TEST[3:0] |  |  |  | Test Mode | Measurement Terminal | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | Normal mode | DSOUT | — |
| 0 | 0 | 0 | 1 | The second integration failure detection mode | DSOUT | 1bit translation result output |
| 0 | 0 | 1 | 0 | DSST failure detection mode | FFTESTCHECK | DSST clock output |
| 0 | 0 | 1 | 1 | DSRST failure detection mode | FFTESTCHECK | DSRST clock output |
| 0 | 1 | 0 | 0 | CMP output fixed clock failure detection mode | FFTESTCHECK | CMP clock output |
| 0 | 1 | 0 | 1 | OCCHP failure detection mode. | FFTESTCHECK | AD chopper clock output |
| 0 | 1 | 1 | 0 | AMPCHP failure detection mode | FFTESTCHECK | AMP chopper clock output |
| 0 | 1 | 1 | 1 | AVCM voltage-failures detection mode | FFTESTCHECK | Analog voltage(880 to 990mV) output |
| 1 | 0 | 0 | 0 | FFT check mode | DSOUT | Disable reset every256 clocks |
| 1 | 0 | 0 | 1 | The second integration failure detection mode and FFT check mode | DSOUT | Disable reset every256 clocks |
| Other |  |  |  | — | — | — |

#### Detail of Direct Test Mode

##### the Second Integration Failure Detection Mode

Among the first integrator and the second integrator of secondary Delta-Sigma ADC for temperature sensors and the mode to confirm you make the first integrator function stop, and you make ADC move only by the second integrator, and whether the second integrator is normally moving.

The choice when putting input in, is possible than the case when input is put in more than a BGR circuit and the outside terminal.

Case of the temperature sensor inner (BGR circuit) occurrence voltage is input to Delta-Sigma ADC part.

Case of chip external signal is input to Delta-Sigma ADC part.

##### FF Check Mode

The mode it's confirmed whether a clock is normally moving at the timing of the inner ADC movement, and to confirm whether FF is normally moving.

It's possible to check the clock at the timing of 5 kinds of movement by changing TEST [3:0].

##### AVCM Voltage (the Integrator AMP Common Mode Voltage Value) Breakdown Detection Mode

It's judged whether a common mode voltage generation circuit inside Delta-Sigma ADC for temperature sensors is normally moving. Output is the DC voltage.

##### FFT Check Mode

FFT (Delta-Sigma ADC frequency response) is checked including SIN wave more than outside in Delta-Sigma ADC for temperature sensors.

##### the Second Integrator FFT Check Mode

Among the first integrator and the second integrator of secondary Delta-Sigma ADC for temperature sensors and the mode to confirm you make the first integrator function stop, and you make ADC move only by the second integrator, and whether the second integrator is normally moving.

FFT (Delta-Sigma ADC frequency response) is checked including SIN wave more than outside in Delta-Sigma ADC for temperature sensors.

## Related RFQ Items

| Document | File name w/ hyperlink |
| --- | --- |
| Requirement Spec. | ACC-BD-17-0020_P2x-EVA1_RequirementSpec_Rev2.0_release.docx |
| RIDERS IP Spec. | [SM] M40PF周辺IP温度センサ設計説明資料兼機能仕様書V00R04 [HM] ARV28TPS0000AKGLTS2K30T55A0_LSI_implement_guide_V00 00 01E_170413.pdf |